Part 2. Spanning Tree Protocol (STP) and Layer 2 Loop Prevention

- Objectives

- To study and understand the basic principles of Spanning Tree Protocol (STP).

- To build a switched network topology and configure IP connectivity between hosts.

- To introduce redundant links between switches and observe how STP prevents Layer 2 loops.

- To analyze STP information and port roles using CLI commands.

- To understand the Root Bridge election process and influence it by modifying bridge priority.

- To observe how STP recalculates the topology and activates a backup path when a link failure occurs.

- Preparation

- Cisco Packet Tracer (Lastest Version)

- Lab Manual with Student instruction and Packet tracer file used in the workshop

- Experiment 1 Topology setup

Please complete the terminal device configuration according to the instructions below：

- Build the network topology in Packet Tracer

- Open Cisco Packet Tracer.

- In the lower-left corner of the Packet Tracer window, locate the Device-Type Selection Box.

- Click Network Devices, then select Switches.

- From the device list, drag 2960 Switch into the workspace.

Place two switches and rename them as: Switch0, Switch1

- In the Device-Type Selection Box, click End Devices.

- Drag PC into the workspace and place six PCs. Rename them from PC0 to PC5.

- Click Connections (the lightning icon).

- Select Copper Straight-Through cable and connect the PCs to the switches according to the following table:

| Device | Interface | Connected Device | Interface |
| --- | --- | --- | --- |
| PC0 | FastEthernet0 | Switch0 | FastEthernet0/1 |
| PC1 |  |  | FastEthernet0/2 |
| PC2 |  |  | FastEthernet0/3 |
| PC3 |  | Switch1 | FastEthernet0/1 |
| PC4 |  |  | FastEthernet0/2 |
| PC5 |  |  | FastEthernet0/3 |

- Use Copper cross-over cable to connect the two switches:

| Device | Interface | Connected Device | Interface |
| --- | --- | --- | --- |
| Swtich0 | FastEthernet0/4 | Swtich1 | FastEthernet0/4 |

After completing the connections, ensure that all link lights turn green, indicating that the connections are active.

- Configure PC IP address

- Click on PC0, navigate to the Desktop tab, and then click on IP Configuration.

- Set the following parameters:

IPv4 Address: 192.168.10.1

Subnet Mask: 255.255.255.0

Default Gateway: 192.168.10.254

- Repeat this operation to configure PC1-PC5 with IPv4 addresses ranging from 192.168.10.2 to 192.168.10.6.

PC Configuration Reference:

| PC No. | IPv4 Address | Subnet Mask | Default Gateway |
| --- | --- | --- | --- |
| PC0 | 192.168.10.1 | 255.255.255.0 | 192.168.10.254 |
| PC1 | 192.168.10.2 |  |  |
| …… | …… |  |  |
| PC5 | 192.168.10.6 |  |  |

Note: You can choose not to follow the table setup. But do configure all six PCs on the same network segment.

- Experiment 2 Introduce Loop and Observe STP Blocking

In this experiment, an additional physical link will be introduced between the two switches to create a Layer 2 loop.

- Add an additional link between the switches

- In Packet Tracer, click Connections (the lightning icon).

- Select Copper cross-over cable.

- Connect an additional link between the two switches using the following interfaces:

| Device | Interface | Connected Device | Interface |
| --- | --- | --- | --- |
| Swtich0 | FastEthernet0/5 | Swtich1 | FastEthernet0/5 |

After completing the connection, there will now be two physical links between the two switches. Since there are now two parallel links between the switches, a Layer 2 loop is created in the network.

- Observe the switch port status

When a loop appears in a Layer 2 network, the switch automatically uses the Spanning Tree Protocol (STP) to prevent network loops.

- Observe the port status indicators on the switch links in Packet Tracer.

- Among the two links connecting the switches, one of the ports will change to Orange/Green/Red

- This indicates that STP has automatically Blocked/Opened one of the redundant links to prevent a network loop.

- Verify STP information using CLI

- Click Switch1 and open the CLI tab.

- Enter the following commands:

Switch> enable

Switch# show spanning-tree

- From the output, observe the following information:

These values show how STP determines the forwarding path and which port should be blocked.

- Practice & Thinking Questions

- Why does one of the switch ports enter the Blocking state after adding a second link?

- What problems might occur if the network operates without STP?(at least two points)

- What is the main purpose of Spanning Tree Protocol in a Layer 2 network?

- Complete the STP Information Table of Switch0:

| Item | Value |
| --- | --- |
| Root Bridge Priority |  |
| Root Bridge MAC Address |  |
| Root Path Cost |  |
| Root Port |  |
| Designated Ports |  |
| Blocking Port |  |

- Experiment 3 Root Bridge Election

In this experiment, we will investigate how Spanning Tree Protocol (STP) elects the Root Bridge in a network and observe how changing the bridge priority can influence the election result.

- Observe the current root bridge

- Click on Switch0 and open the CLI tab.

- Enter the following commands:

Switch> enable

Switch# show spanning-tree

- Repeat the same command on Switch1 and find the root bridge.

- Change & Verify the bridge priority

STP selects the Root Bridge based on the Bridge ID, which is composed of:

Bridge ID = Priority + MAC Address

A switch with the lowest Bridge ID will become the Root Bridge.

- On Switch0, enter the following commands:

Switch> enable

Switch# configure terminal

Switch(config)# spanning-tree vlan 1 priority 4096

This command lowers the bridge priority of Switch0, increasing its chance of becoming the Root Bridge.

- On Switch0, run the following command again:

Switch# show spanning-tree

- Check whether the output now shows:

This bridge is the root

- Run the same command on Switch1 and observe how the Root Bridge information changes.

- Practice & Thinking Questions

- What factor determines which switch becomes the Root Bridge?

- After changing the Root Bridge, does the Blocking port change? Explain why.

- Experiment 4 Link Failure Recovery

In this experiment, we will observe how Spanning Tree Protocol (STP) responds to a link failure and automatically activates a backup path.

- Disconnect the active link and observe STP reconvergence

- In Packet Tracer, select the Delete tool.

- Identify the port that is currently in the Blocking state.

- Remove the currently active link between the two switches. This link is the one whose ports are in the Forwarding state (Green).

- Identify the port that was previously in the Blocking state. After a short period of time, the port state will change from Blocking (Orange) to Forwarding (Green).

- Click Switch0 and open the CLI tab.

- Enter the following command to view the STP information:

Switch> enable

Switch# show spanning-tree

Observe the changes in the port roles and port states.

- Practice & Thinking Questions

- What happens to the previously blocked port after the active link fails?

- What is the advantage of having redundant links in a network?