## Part 3. Virtual LAN(VLAN)Management, Link Aggregation, and Inter-VLAN Routing

### Objectives

- To study and practice the principles of VLAN.

- Create multiple VLANs and assign corresponding ports.

- Create a trunk line between 2 switches for paired VLANs.

- By implementing the port security understands Layer 2 Security.

- Understand the implementation of EtherChannel.

- implement Router-on-a-Stick for communication between different IP subnets.

### Preparation

- 2 switches (Like 2960).

- 6 PC.

- Each switch is connected to 3 PCs.

- Connect the 2 switches.

### Description of laboratory experiment set-up

### Experiment 1 – Basic Configuration

#### Basic configuration

Please complete the terminal device configuration according to the instructions below：

- Click on PC0, navigate to the Desktop tab, and then click on IP Configuration.

- Set the IPv4 Address to 192.168.10.1, the Subnet Mask to 255.255.255.0 and the Default Gateway to 192.168.10.254.

- Repeat this operation to configure PC1-PC5 with IP addresses ranging from 192.168.10.2 to 192.168.10.6.

PC Configuration Reference:

| PC No. | IPv4 Address | Subnet Mask | Default Gateway |
| --- | --- | --- | --- |
| 0 | 192.168.10.1 | 255.255.255.0 | 192.168.10.254 |
| 1 | 192.168.10.2 | 255.255.255.0 | 192.168.10.254 |
| …… | …… | …… | …… |
| 5 | 192.168.10.6 | 255.255.255.0 | 192.168.10.254 |

Note: You can choose not to follow the table setup. But do configure all six PCs on the same network segment.

#### Create VLANs and name them

Repeat the following commands on both switches (Switch0 and Switch1):

Switch> enable

Switch# configure terminal

Switch(config)# vlan 10

Switch(config-vlan)# name EEE

Switch(config-vlan)# vlan 20

Switch(config-vlan)# name NIE

Switch(config-vlan)# vlan 30

Switch(config-vlan)# name CCDS

Switch(config-vlan)# exit

Or you can choose to change the config on both switches (Switch0 and Switch1):

#### Assign access port

Execute on Switch0 and Switch1 respectively:

- Assign PC0 / PC3 to VLAN 10 (interface Fa0/1):

Switch(config)# interface FastEthernet0/1

Switch(config-if)# switchport mode access

Switch(config-if)# switchport access vlan 10

- Assign PC1 / PC4 to VLAN 20 (interface Fa0/2):

Switch(config)# interface FastEthernet0/2

Switch(config-if)# switchport mode access

Switch(config-if)# switchport access vlan 20

- Assign PC2 / PC5 to VLAN 30 (interface Fa0/3):

Switch(config)# interface FastEthernet0/3

Switch(config-if)# switchport mode access

Switch(config-if)# switchport access vlan 30

Or change according FastEthernet config:

#### Configure trunk

Execute on Switch0 and Switch1 respectively:

Switch(config)# interface FastEthernet0/4

Switch(config-if)# switchport mode trunk

Or change according ethernet config:

Note: After executing this command, the link lights in Packet Tracer may temporarily turn red and then green, indicating that the Spanning Tree Protocol (STP) is converging again.

#### View VLAN & trunk & interfaces information

Enter “enable” at the “Switch> ” (At Switch’s CLI interface),

- To view VLAN allocation, input: “show vlan brief”:

- Check whether Trunk is enabled: “show interfaces trunk”

- Check interface status: “show ip interface brief”

#### Practice & Thinking Questions

- Test 1 (VLAN Cross-Switch): Please ping the VLAN 10 host of Switch 1 from the VLAN 10 host of Switch 0. What is the result? What does this indicate about the role of the Trunk link?

- Test 2 (Communication across Different VLANs): Please ping the host on VLAN 20 of Switch 0 from the host on VLAN 10 of Switch 0. What is the result? Explain why this result occurs, taking into account the MAC address table (show mac address-table).

- Test 3: Change one end of the Trunk link between two switches back to Access mode, observe the changes in network communication, and consider what consequences would arise from a mismatch in negotiation between the two ends.

- Q 1 (Broadcast Domain Isolation): When no VLAN is configured, if a PC sends an ARP broadcast request, which devices in the network can receive it? After configuring VLANs 10, 20, and 30, if a PC within VLAN 10 sends an ARP broadcast, who else can receive it? Please briefly describe how VLAN improves network performance and security.

- Q 2 (The Essence of Trunk): If we do not configure a Trunk interface, but instead use three network cables to connect the VLAN 10, 20, and 30 interfaces of two switches respectively, can we achieve the same communication effect? If so, why do we still use Trunk technology?

- Q 3 (No routing state): In the experiment, different VLANs cannot communicate with each other. If we want VLAN 10 and VLAN 20 to communicate with each other, what type of device do we need to introduce into the network topology? (To pave the way for the next stage of the single-arm router/layer 3 switch experiment)

Note: Congratulations on completing Experiment 1！

The following 3 experiments are based on this experiment (Experiment 1). Please save 		the .pkt file current as a template for subsequent experimental operations.

### Experiment 2 -- EtherChannel and Spanning Tree Protocol （STP）Observation

#### Upgrade Topology and Observe the initial state of STP

Connect an additional crossover cable between two 2960 switches. After connecting the cable without further configuration, one port on one of the links will turn orange (Blocking state).

Enter show spanning-tree in the privileged mode to observe the root bridge election results and specific port statuses (blocked ports are displayed as BLK)

#### Configure EtherChannel

In the global configuration mode of Switch 0 and Switch 1, enter the following identical configuration commands:

enable

configure terminal

# 1.Enter the two connected physical interfaces simultaneously.

interface range fastEthernet 0/4-5

# 2.Bundle these two ports into Channel Group 1 and set them to active negotiation mode.

channel-group 1 mode active

exit

# Enter the newly created logical interface Port-Channel 1.

interface port-channel 1

#Configure this logical interface as Trunk mode to carry all VLANs.

switchport mode trunk

End

Use the Fast Forward Time feature in Packet Tracer to manually accelerate the convergence of STP and LACP (Link Aggregation Control Protocol). Once the network converges, all four port indicators between the switches turn green. Communication between PC1 and PC4 remains normal.

#### Verification Procedures

Execute the following three core commands in the privileged mode (Switch#) of Switch 0 or Switch 1:

- Check Aggregation Summary Status

Command: Switch# show etherchannel summary

Check Group Status if Po1 is marked as (SU). S stands for Layer 2, and U stands for In Use, which signifies success.Check Ports Status if physical ports (e.g., Fa0/4, Fa0/5) are marked as (P), indicating they are successfully bundled into the Port-channel.

- Check Spanning Tree Protocol (STP) Status

command：Switch# show spanning-tree

The original physical ports Fa0/4 and Fa0/5 should disappear from the list, replaced by the logical interface Po1 in the FWD (Forwarding) state.

- Final Connectivity Test

Perform a continuous Ping test from PC0 (VLAN 10) to PC3 (VLAN 10). Ping responses are normal with 0% loss.

Why did the orange port (blocked by STP) turn green after configuring EtherChannel?

### Experiment 3 -- Port Security Configuration

#### Port security configuration command (CLI)

Please click on Switch0 to enter the CLI tab, and then enter the following commands 	in sequence:

Switch> enable

Switch# configure terminal

Switch(config)# interface FastEthernet0/3

# The port must first be set to Access mode (Port Security does not support Trunk or dynamic ports)

Switch(config-if)# switchport mode access

# Enable port security function

Switch(config-if)# switchport port-security

# Limit the maximum number of MAC addresses allowed to be learned to 1

Switch(config-if)# switchport port-security maximum 1

# Set to Sticky, permanently binding the currently connected PC's MAC address to the configuration

Switch(config-if)# switchport port-security mac-address sticky

# Set the violation triggering action (default is shutdown, which means closing the port)

Switch(config-if)# switchport port-security violation shutdown

#### Ensure that the switch "captures" the identity of legitimate hosts

Ping other PCs in the net. Making sure that the switch remember the MAC address.

You can confirm that the Sticky MAC Addresses has changed to 1 by entering "			show port-security interface fa0/3" on the switch.

#### Simulating "hacker" access

Disconnect: Use the delete tool (shortcut key Del) to remove the connection b				etween Switch0 and the original PC2.

Connect a new device: Add a new laptop or PC and connect it to Fa0/3 of Switch0.

Trigger: When configuring the IP address for the hacker's PC, you will notice that 	the light on the Fa0/3 interface instantly changes from green to red.

At this time, you can view the following information by checking the CLI interface of the switch:

#### Phenomenon observation and recovery

When a port is disabled due to violations, it enters a special state known as "err-d		isabled".

- View violation records

Enter the follow commands at Switch’s CLI tab(Switch#):

Switch# show port-security interface FastEthernet0/3

You will see in the output:

Port Status:

Security Violation Count:

Last Source Address: [The MAC address of the hacker's PC will be displayed here]

- Restore interface

First, reconnect the originally legal PC.

Enter the following command at switch’s CLI tab:

Switch# configure terminal

Switch(config)# interface FastEthernet0/3

Switch(config-if)# shutdown

Switch(config-if)# no shutdown

It can be observed that the original PC has restored normal connection.

Question: If you directly enter "not shutdown" instead of "shutdown" first, will the original PC be able to restore normal connection? What is the status of the interface? Why?

#### Auto Recovery

In real-world production environments, manually closing and reopening hundreds of 	ports can be extremely cumbersome. Cisco provides an automatic recovery function:

Switch(config)# errdisable recovery cause psecure-violation

# Set the waiting time for automatic recovery (e.g. 30 seconds)

Switch(config)# errdisable recovery interval 30

### Experiment 4 Router-on-a-Stick Configuration

#### Upgrade Topology

Use a straight-through cable to connect Switch0 (Fa0/10) to Router 2911. (GigabitEthernet 0/0). Initially, both ends of the cable will appear red; this is normal because router interfaces are disabled by default.

#### Configure Router-on-a-Stick

- Switch Side: Backbone Link Configuration

To allow the link connecting to the router to carry tagged packets for multiple VLANs, the port must be set to Trunk mode. Enter the CLI for Switch0:

Switch>enable

#User Mode → Privileged Mode: Gain authorization to view and modify configurations.

Switch#configure terminal

#Privileged Mode → Global Configuration Mode: Prepare to modify switch parameters.

Switch(config)#interface fastEthernet 0/10

#Interface Configuration Mode: Specify the Fa0/10 port connected to the router.

Switch(config-if)#switchport mode trunk

#Configure Trunking: Allow the link to carry data for multiple VLANs (10 and 20).

Switch(config-if)#no shutdown

#Activate Interface: Ensure the physical port is turned on.

Switch(config-if)#exit

#Exit Interface: Return to the previous Global Configuration level.

- Router Side: Sub-interface and Gateway Configuration

The router provides an independent Layer 3 entry point for each VLAN by dividing logical sub-interfaces.Enter the CLI for the Router:

Router> enable

# Enter privileged mode: Ready to start configuring the router.

Router# configure terminal,

# Enter global configuration mode: Ready to enter configuration commands.

Router(config)#interface gigabitEthernet 0/0,

# Physical interface mode: Enter the main physical interface to perform the enable    operation.

Router(config-if)# no shutdown

# Enable the physical master switch: The connection lines in the topology diagram will   change from red to green at this point.

Router(config-if)#exit

# Return to global configuration mode: Ready to create subinterfaces.

--- Subinterface 10 ---, ,

Router(config)#interface gigabitEthernet 0/0.10,

#Enter subinterface mode: Create a logical interface dedicated to VLAN 10.

Router(config-subif)#encapsulation dot1Q 10

# Encapsulation protocol: Bind the 802.1Q protocol and specify that it belongs to VLAN 10.

Router(config-subif)#ip address 192.168.10.254 255.255.255.0,

#Configure Gateway IP: Provide a routing exit for VLAN 10 hosts

Router(config-subif)#exit

#Return: Prepare to configure the next VLAN.

--- Sub-interface 20 ---, ,

Router(config)#interface gigabitEthernet 0/0.20,

#Sub-interface Mode: Create a logical interface dedicated to VLAN 20.

Router(config-subif)#encapsulation dot1Q 20,

#Encapsulation Protocol: Bind the 802.1Q protocol to VLAN ID 20.

Router(config-subif)#ip address 192.168.20.254 255.255.255.0,

#Configure Gateway IP: Provide a routing exit for VLAN 20 hosts.

Router(config-subif)#end,

#Finish Configuration: Exit back directly to Privileged Mode.

#### Verification Procedures

- Terminal Configuration Verification

Click the PC icon, select the Desktop tab, and click IP Configuration to enter details:

PC0 (VLAN 10): IP 192.168.10.1, Default Gateway must point to 192.168.10.254.

PC3 (VLAN 20): IP 192.168.20.1, Default Gateway must point to 192.168.20.254.

- Connectivity Test (Ping Test)

Open the Command Prompt on PC0 and enter ping 192.168.20.1. Due to ARP requests, the first 1-2 packets may "Timed out," followed by "Reply from...". Successful replies prove cross-segment communication is successful.

- Routing Table Verification

Enter show ip route on the router CLI. You should see two routes marked with C (Connected) pointing to Gi0/0.10 and Gi0/0.20 respectively.

Why is Trunk mode mandatory for the Switch-Router link?

What happens if the Default Gateway on the PC is configured incorrectly?

What are the limitations of the Router-on-a-Stick approach in a high-traffic environment?